- Esp32 web hosting and control led using mobile phone

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "ESP32_LED_Control";

const char* password = "12345678";

WebServer server(80);

// ---------------- PWM ----------------

const int LED_PIN = 18;

const int PWM_FREQ = 5000;

const int PWM_RESOLUTION = 8;   // 8-bit (0-255)

// ---------------- HTML ----------------

const char webpage[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>ESP32 LED Brightness</title>

<style>

body{

font-family:Arial;

text-align:center;

background:#f5f5f5;

margin-top:40px;

}

h1{

color:#1976D2;

}

.slider{

width:80%;

}

.value{

font-size:28px;

color:#444;

margin:20px;

}

</style>

</head>

<body>

<h1>ESP32 LED Controller</h1>

<div class="value">

Brightness :

<span id="brightness">0</span>

</div>

<input type="range"

min="0"

max="255"

value="0"

class="slider"

id="slider"

oninput="updateBrightness(this.value)">

<script>

function updateBrightness(value)

{

document.getElementById("brightness").innerHTML = value;

var xhr = new XMLHttpRequest();

xhr.open("GET","/set?value="+value,true);

xhr.send();

}

</script>

</body>

</html>

)rawliteral";

// ---------------- Handlers ----------------

void handleRoot()

{

server.send(200, "text/html", webpage);

}

void handleBrightness()

{

if(server.hasArg("value"))

{

int value = server.arg("value").toInt();

value = constrain(value,0,255);

ledcWrite(LED_PIN, value);

Serial.print("Brightness : ");

Serial.println(value);

}

server.send(200,"text/plain","OK");

}

void setup()

{

Serial.begin(115200);

// Configure PWM

if(!ledcAttach(LED_PIN, PWM_FREQ, PWM_RESOLUTION))

{

Serial.println("Failed to configure PWM!");

while(1);

}

ledcWrite(LED_PIN,0);

// Create Access Point

WiFi.mode(WIFI_AP);

WiFi.softAP(ssid,password);

Serial.println();

Serial.println("=================================");

Serial.println("Access Point Started");

Serial.print("SSID : ");

Serial.println(ssid);

Serial.print("Password : ");

Serial.println(password);

Serial.print("IP Address : ");

Serial.println(WiFi.softAPIP());

Serial.println("=================================");

server.on("/",handleRoot);

server.on("/set",handleBrightness);

server.begin();

Serial.println("HTTP Server Started");

}

void loop()

{

server.handleClient();

}

- Connect esp32 to wifi and upload data to Firebase cloud database

#include <WiFi.h>

#include <HTTPClient.h>

#include <time.h>

// =========================

// WiFi Credentials

// =========================

const char* ssid = "Wifi Name ";

const char* password = "Password";

// =========================

// Firebase Realtime Database URL

// (No trailing slash)

// =========================

const char* firebaseHost =

"https://esp32-ir-e27ee-default-rtdb.asia-southeast1.firebasedatabase.app";

// =========================

// IR Sensor Pin

// =========================

const int IR_PIN = 27;

// =========================

// Time Settings (IST)

// =========================

const char* ntpServer = "pool.ntp.org";

const long gmtOffset_sec = 19800;

const int daylightOffset_sec = 0;

String previousStatus = "";

void setup()

{

Serial.begin(115200);

pinMode(IR_PIN, INPUT);

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

// Synchronize time

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);

struct tm timeinfo;

Serial.print("Synchronizing Time");

while (!getLocalTime(&timeinfo))

{

Serial.print(".");

delay(500);

}

Serial.println("\nTime synchronized!");

}

void loop()

{

// LOW = Object Detected

// HIGH = Not Detected

String status;

if (digitalRead(IR_PIN) == LOW)

status = "Object Detected";

else

status = "Not Detected";

// Upload only if state changes

if (status != previousStatus)

{

previousStatus = status;

struct tm timeinfo;

if (getLocalTime(&timeinfo))

{

char timeString[30];

strftime(timeString,

sizeof(timeString),

"%d %B %Y %I:%M:%S %p",

&timeinfo);

Serial.println("--------------------------------");

Serial.print("Status : ");

Serial.println(status);

Serial.print("Time   : ");

Serial.println(timeString);

// Firebase keys cannot contain spaces or colons

String key = String(timeString);

key.replace(" ", "_");

key.replace(":", "-");

if (WiFi.status() == WL_CONNECTED)

{

HTTPClient http;

String url = String(firebaseHost) +

"/detections/" +

key +

".json";

http.begin(url);

http.addHeader("Content-Type", "application/json");

// Store only the status as the value

String json = "\"" + status + "\"";

int response = http.PUT(json);

Serial.print("HTTP Response: ");

Serial.println(response);

http.end();

}

}

}

delay(100);

}

- Blink built in led

const int LED_PIN = 2;

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, HIGH); // LED ON

delay(500);

digitalWrite(LED_PIN, LOW);  // LED OFF

delay(500);

}

- Ultrasonic distance measurement

#define TRIG_PIN 5

#define ECHO_PIN 18

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

}

void loop() {

// Clear trigger

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Send 10µs pulse

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Measure echo duration

long duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance (cm)

float distance = duration * 0.0343 / 2.0;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(500);

}